- 首先，修正前次作業覺得不合理的地方。

- 在做 Design Project 2 的時候發現 Design Project 1 的平面圖畫錯了，應該要長下面這樣。

- 接下來修正 Design Project 2 尺寸的問題。

  - BRB 尺寸前一次做太大了，後來發現 BRB 由於不會發生 buckling，所以 ETABS 做 BRB 的時候，未支撐的長度可以設定很小，讓 BRB 不會發生 buckle，如下圖。

  - 之後 BRB 的尺寸就變得比較合理了，並且由於之前 BRB 尺寸太大造成的梁柱斷面太大也要一起修改。

  - 當 BRBF 變得合理之後，前一次發現 BRB 吃掉大部分的側力的情況就沒有發生了，反而是 EBF 吃到比較多的力。( 這個地方感覺還有點問題，不知道為什麼 EBF 會吃到比較多的力。推測是因為 EBF 在中間，所以吃到比較多 DL，所以也吃到比較多地震力，但還不確定是不是這樣，需要探討。)

  - 所以 EBF 的尺寸也需要重新設計。EBF Brace 選用 BOX。並重新調整梁柱尺寸。

  - 調整完之後，發現週期比較正常了，不會像之前一樣，因為勁度太大而週期太小，地震力也稍微降下來一點，不再處於平台段。第二模態也不再是扭轉，而是 Y 向的震動。

  - 大致修正完成之後，進入 Design Project 3 進行設計。

- 從 0.32g 正規化到 0.08g

- 然後乘以 9.81 輸入 ETABS

- 可以發現正規化到 0.08g 的反應譜，和考慮結構任性的反應譜差不多大。所以正規化到 0.08g 的反應譜是合理的。

Case1 DEAD

Area uniform load

RF: Average concrete slab weight + Ceiling, air-condition piping and floor finishing = 0.28 + 0.15 = 0.43 tf/m2

Specially, in the center of the roof, 8m×8m is water tank so the weight of center of the roof = 0.28 + 0.15 + 0.1 = 1.43 tf/m2

1F – 3F: Average concrete slab weight + Ceiling, air-condition piping and floor finishing + Partition walls = 0.28 + 0.15 + 0.1 = 0.53 tf/m2

Line uniform load

RF: Exterior walls = 0.1 × 1.4 = 0.14 tf/m

1F - 3F: Exterior walls = 0.1 × 4 * 0.7(assume 30% is window, so no wall weight) = 0.28 tf/m

Case2 SELF WEIGHT

Area uniform load

The average weight of steel frame (including gravity beams, girders) = 0.45 tf/m2

Others for ETABS auto calculate.

Case3 LIVE

Area uniform load

Live Load = 0.3 tf/m2

RF:

- Live Load Reduction Factor Setting

- Load Combinations

- MRF DCR

- BRBF DCR

- EBF DCR

- Check Inter Story Drifts

- Average of the lateral resisting frame weight

- Frame Member Size

  - MRF

  - BRBF

  - EBF

- Lateral Force

  - X 向

  - Y 向

- Story Shear

  - X 向

  - Y 向

- Overturning Moment

  - X 向

  - Y 向

- Inter-story Drifts

  - X 向

  - Y 向

- Displacement

  - X 向

  - Y 向

- 小梁和大梁都按照力，讓 ETABS AUTO-SELECT