Seismic Resistant Design of Steel Structures

Design Project (Assignment #3)

R05521203

張世昇

6/5/2017

- DBE acceleration response spectrum

根據耐震設計規範，依照本次設計作業所提供的場址和結構系統繪製加速度反應譜，並將加速度反應譜正規化至PGA = 0.08g，完成後將加速度反應譜資料匯出至文字檔，在匯入ETABS中的Response Spectrum Function Definition以進行動力分析。

Acceleration Response Spectrum:

Response Spectrum Function Defintion:

Response Spectrum Case(x/y direction):

Analytical result:

| Direction | Base Shear (kgf) |  | β | 1/β |
| --- | --- | --- | --- | --- |
|  | Static | Dynamic |  |  |
| Longitudinal direction | 635771.3 | 390340.8 | 0.614 | 1.629 |
| Transverse direction | 738835.8 | 512815.7 | 0.694 | 1.441 |

CQC base shear in X direction is 390340.8 kgf → βx = 0.614

CQC base shear in Y direction is 512815.7 kgf → βy = 0.694

- Apply Load Case

根據Assignment#1和Assignment#3所給的資料施加載重。

首先，依照施加載重之需求先繪製相關樓版及梁，但為了不影響本身結構周期在繪製過程中先不指定斷面。並依照題目要求新增3個Load Case (DEAD LOAD、LIVE LOAD、SELF WEIGHT)。

Case1 DEAD

Area uniform load

RF: Average concrete slab weight + Ceiling, air-condition piping and floor finishing = 280 + 150 = 430kgf/m2

Specially, in the center of the roof(9m×9m is water tank so the weight of center of the roof = 280 + 150 + 1000 = 1430kgf/m2

1F – 3F: Average concrete slab weight + Ceiling, air-condition piping and floor finishing + Partition walls = 280 + 150 + 100 = 530kgf/m2

Line uniform load

RF: Exterior walls = 120 × (1.4 + 1.9) = 396kgf/m

1F – 3F: Exterior walls = 120 × 3.8 = 456kgf/m

Case2 SELF WEIGHT

Area uniform load

The average weight of steel frame (including columns, girders, beams and braces) = 80 = 80kgf/m2

Case3 LIVE

Area uniform load

Live Load = 250kgf/m2

以RF為例

Uniform Load                     Line Load

- Check DCR

- Live Load Reduction Factor Setting

Options/Preference/Live Load Reduction

- Load combinations

| CASE | Define Combination |
| --- | --- |
| COMB1 | 1.4DEAD+1.4SELFWEIGHT |
| COMB2 | 1.2DEAD+1.2SELFWEIGHT+1.6LIVE |
| COMB3 | 1.2DEAD+1.2SELFWEIGHT+0.5LIVE+1.0EQX |
| COMB4 | 1.2DEAD+1.2SELFWEIGHT+0.5LIVE+1.0EQY |
| COMB5 | 1.2DEAD+1.2SELFWEIGHT+0.5LIVE-1.0EQX |
| COMB6 | 1.2DEAD+1.2SELFWEIGHT+0.5LIVE-1.0EQY |
| COMB7 | 1.2DEAD+1.2SELFWEIGHT+0.5LIVE+1/βSPECX |
| COMB8 | 1.2DEAD+1.2SELFWEIGHT+0.5LIVE+1/βSPECY |
| COMB9 | 1.2DEAD+1.2SELFWEIGHT+0.5LIVE-1/βSPECX |
| COMB10 | 1.2DEAD+1.2SELFWEIGHT+0.5LIVE-1/βSPECY |

- Check DCR of MRF

- Check DCR of BRBF

ETABS預設之壓力強度是以挫曲強度進行計算，所以我們需要修改ㄧ些設計部分的設定。

Design/Steel Frame Design/View or Revise Overwrites

- Check DCR of EBF

- Check Inter Story Drifts

| Inter Story Drifts |  |
| --- | --- |
| Longitudinal static lateral load | Transversal static lateral load |
| Longitudinal dynamic spectrum load | Transversal dynamic spectrum load |

- Average of the lateral resisting frame weight

| Group | SelfMass | SelfWeight | TotalMassX | TotalMassY | TotalMassZ |
| --- | --- | --- | --- | --- | --- |
| ALL | 22881.84 | 224575.2 | 339329.6 | 339329.6 | 0 |
| 4F | 5604.545 | 55006.15 | 81193.54 | 81193.54 | 0 |
| 3F | 5309.501 | 52110.42 | 85595.74 | 85595.74 | 0 |
| 2F | 6131.419 | 60177.19 | 86417.66 | 86417.66 | 0 |
| 1F | 6131.419 | 60177.19 | 86417.66 | 86417.66 | 0 |

| Total structure weight (kgf) | Average structure weight (kgf/m2) |
| --- | --- |
| 224575.224 | 47.259 |

- Plot static and dynamic lateral force, story shear and overturning moment distributions. And show the lateral displacements and the inter-story drifts under the static and dynamic (before and after scaling) loads.

- Frame member size

MRF:

| Story | Beam | Wide Flange Column | Box Column at corner |
| --- | --- | --- | --- |
| RF | H600×350×19×40 | H720×350×22×50 | HSS750×750×30 |
| 4F | H600×350×19×40 | H720×350×22×50 | HSS750×750×30 |
| 3F | H700×350×19×40 | H720×350×22×50 | HSS750×750×32 |
| 2F | H700×350×19×40 | H720×350×22×50 | HSS750×750×32 |

BRBF:

| Story | Beam | Wide Flange Column | Brace |
| --- | --- | --- | --- |
| RF | H390×300×10×16 | H390×300×10×16 | Rec55×55 |
| 4F | H390×300×10×16 | H390×300×10×16 | Rec55×55 |
| 3F | H390×300×10×16 | H442×400×22×36 | Rec65×65 |
| 2F | H390×300×10×16 | H442×400×22×36 | Rec65×65 |

EBF:

| Story | Beam and Link | Wide Flange Column | Brace |
| --- | --- | --- | --- |
| RF | H442×400×22×36 | H390×300×10×16 | HSS300×300×16 |
| 4F | H442×400×22×36 | H390×300×10×16 | HSS300×300×16 |
| 3F | H450×400×25×40 | H600×350×19×40 | HSS300×300×16 |
| 2F | H450×400×25×40 | H600×350×19×40 | HSS300×300×16 |

- Lateral Force

| Longitudinal Direction Lateral Force(kgf) |  |  |  |
| --- | --- | --- | --- |
| Story | EQX | SPECX(Before Scaling) | SPECX(After Scaling) |
| RF | 245167.4 | 163061.05 | 265698.3134 |
| 4F | 195301.9 | 112972.94 | 184082.7078 |
| 3F | 130201.3 | 76280.06 | 124293.8353 |
| 2F | 65100.65 | 37863.54 | 61696.39357 |

| Transverse Direction Lateral Force(kgf) |  |  |  |
| --- | --- | --- | --- |
| Story | EQY | SPECY(Before Scaling) | SPECY(After Scaling) |
| RF | 284911.3 | 205034.43 | 295583.2805 |
| 4F | 226962.2 | 155241.04 | 223799.7582 |
| 3F | 151308.1 | 103687.41 | 149478.6256 |
| 2F | 75654.07 | 48538.2 | 69974.00575 |

- Story Shear

| Longitudinal Direction Story Shear(kgf) |  |  |  |
| --- | --- | --- | --- |
| Story | EQX | SPECX(Before Scaling) | SPECX(After Scaling) |
| RF | 245167.4 | 163061.05 | 265698.3134 |
| 4F | 440469.3 | 276033.99 | 449781.0212 |
| 3F | 570670.6 | 352314.05 | 574074.8564 |
| 2F | 635771.3 | 390177.59 | 635771.25 |

| Transverse Direction Story Shear(kgf) |  |  |  |
| --- | --- | --- | --- |
| Story | EQY | SPECY(Before Scaling) | SPECY(After Scaling) |
| RF | 284911.3 | 205034.43 | 295583.2805 |
| 4F | 511873.5 | 360275.47 | 519383.0387 |
| 3F | 663181.6 | 463962.88 | 668861.6643 |
| 2F | 738835.7 | 512501.08 | 738835.67 |

- Overturning Moment

| Longitudinal Direction Overturning Moment(kgf-m) |  |  |  |
| --- | --- | --- | --- |
| Story | EQX | SPECX(Before Scaling) | SPECX(After Scaling) |
| RF | 0 | 0 | 0 |
| 4F | 931636.1 | 619631.991 | 1009653.593 |
| 3F | 2605419 | 1645631.127 | 2681458.355 |
| 2F | 4773968 | 2939472.86 | 4789696.749 |

| Transverse Direction Overturning Moment(kgf-m) |  |  |  |
| --- | --- | --- | --- |
| Story | EQY | SPECY(Before Scaling) | SPECY(After Scaling) |
| RF | 0 | 0 | 0 |
| 4F | 1082663 | 779130.821 | 1123216.447 |
| 3F | 3027782 | 2132258.125 | 3073922.03 |
| 2F | 5547872 | 3863416.104 | 5569607.045 |

- Lateral Displacements

| Longitudinal Direction Lateral Displacements(m) |  |  |  |
| --- | --- | --- | --- |
| Story | EQX | SPECX(Before Scaling) | SPECX(After Scaling) |
| RF | 0.0443 | 0.027 | 0.0440 |
| 4F | 0.0348 | 0.0212 | 0.0345 |
| 3F | 0.0219 | 0.0134 | 0.0218 |
| 2F | 0.0084 | 0.0051 | 0.0083 |

| Transverse Direction Lateral Displacements(m) |  |  |  |
| --- | --- | --- | --- |
| Story | EQY | SPECY(Before Scaling) | SPECY(After Scaling) |
| RF | 0.0243 | 0.0168 | 0.0242 |
| 4F | 0.0192 | 0.0133 | 0.0192 |
| 3F | 0.0124 | 0.0086 | 0.0124 |
| 2F | 0.0051 | 0.0035 | 0.0050 |

- Inter-Story Drifts

| Longitudinal Direction Inter-Story Drift(rad) |  |  |  |
| --- | --- | --- | --- |
| Story | EQX | SPECX(Before Scaling) | SPECX(After Scaling) |
| RF | 0.002501 | 0.001574 | 0.002565 |
| 4F | 0.003372 | 0.002081 | 0.003391 |
| 3F | 0.003572 | 0.002185 | 0.003560 |
| 2F | 0.002203 | 0.001346 | 0.002193 |

| Transverse Direction Inter-Story Drift(rad) |  |  |  |
| --- | --- | --- | --- |
| Story | EQY | SPECY(Before Scaling) | SPECY(After Scaling) |
| RF | 0.001399 | 0.000987 | 0.001423 |
| 4F | 0.001864 | 0.001302 | 0.001877 |
| 3F | 0.001922 | 0.001338 | 0.001929 |
| 2F | 0.001334 | 0.000926 | 0.001335 |

- Design gravity gravity beam and gravity girder

- ETABS

先繪製小梁斷面設計先選擇Auto-Select，並繪製樓版，將設計部分設定為複合梁。

Design/Overwrite Frame Design Procedure

完成設定後，接下來執行Design/Composite Beam Design/Start Design Using Similarity進行設計。

Result:

| Type | Section |
| --- | --- |
| Gravity Beam | H350×350×12×19 |
| Gravity Girder | H350×350×12×19、H600×400×19×40 |

剪力釘得直徑為0.0191m，極限強度Fu為45699526.kgf/m2

- Hand calculations

以下方標記之gravity beam and gravity girder為例

Gravity Beam

(H350X350X12X19 in the interior which is showed in the above picture)

bE=minL12+bfbf+6tsbf+S12=min120012+3535+6×7.545+3002=80cm

Cmax=0.85×fc'×bE×ts=0.85×280×80×7.5=142800 kgf

Tmax=AS×fy=35×35-35-2×1.935-1.2×3500=596540kgf

qult=0.6fultAstud=0.6×4569.95×π×1.90522=7815.244kgf

∴N=minCmaxqult，Tmaxqult=min1428007815.244，5965407815.244=min18.272，76.33=min19，77=19

Take 2N shear studs equal to 38 for interior gravity beam.

Gravity Girder

(H600X400X19X40 in the interior which is showed in the above picture)

bE=minL4bf+16tsS1+S22=min900440+16×7.51200+9002=160cm

Cmax=0.85×fc'×bE×ts=0.85×280×160×(7.5+7.52)=428400 kgf

Tmax=AS×fy=60×40-60-2×440-1.9*3500=1465800kgf

qult=0.6fultAstud=0.6×4569.95×π×1.90522=7815.244kgf

∴N=minCmaxqult，Tmaxqult=min4284007815.244，14658007815.244=min54.816，187.557=min36，187=54

Take 3N shear studs equal to 162 for interior gravity girder.

Comparison

|  | ETABS | Hand Calculation |
| --- | --- | --- |
| Gravity Beam | 29 | 38 |
| Gravity Girder | 10 | 162 |

- Check the average weight of gravity beam and gravity girder

| Type | Section | number | Total weight(kgf) | Average weight(kgf/m2) |
| --- | --- | --- | --- | --- |
| Gravity Beam | H350×350×12×19 | 96 | 140989.454 | 29.67 |
| Gravity Girder | H600×400×19×40 | 76 | 253159.946 | 53.27 |
| Sum |  | 172 | 394149.4 | 82.94 |

- Design the Gravity Columns

Consider the amplification factor of 1.4αy Ra to amplify the lateral force. Therefore, we should consider the P-Δeffect.

1.4αy Ra = 1.4×1×3.533 = 4.95

因此利用ETABS內建之A-GravCol section繪製 Gravity Column.

| Story | section |
| --- | --- |
| 3F-4F | H400×408×21×21 |
| 1F-2F | H450×400×25×40 |

Check DCR of Gravity Column:

The weight of Gravity Column:

| Story | section | number | Total Weight(kgf) | Average Weight(kgf/m2) |
| --- | --- | --- | --- | --- |
| 3F-4F | H400×408×21×21 | 4 | 2935.499 | 1.235 |
| 1F-2F | H450×400×25×40 | 4 | 4911.551 | 2.067 |

- Average Total Weight of Steel

| Type | Average Weight(kgf/m2) |
| --- | --- |
| Lateral Load Resisting System | 47.26 |
| Gravity Beam | 29.67 |
| Gravity Girder | 53.27 |
| Gravity Column | 2.07 |
| Sum | 132.27 |

在初步設計時我們假設average weight of steel frame = 80 kgf/m2，設計結果為132.27 kgf/m2，可能原因為設計過度保守，導致鋼材用量過多。但在後續設計過程中我們除了考慮原先假設之自重80 kgf/m2之外也加入鋼材自重，所以在安全上是沒有疑慮的，所以不用重新分析。

Results of vibration period

| Mode | Period (sec) |  |
| --- | --- | --- |
| 1 | 0.9284 |  |
| 2 | 0.6259 |  |
| 3 | 0.4744 |  |
| 4 | 0.2834 |  |
| 5 | 0.2167 |  |
| 6 | 0.1565 |  |